# Network Devices & Local Area Network (LAN)

## Introduction

A network device is a hardware component used to connect computers, servers, printers, and other electronic devices to form a communication network. These devices enable communication by transmitting, receiving, routing, and managing data across the network.

# 1. Network Interface Card (NIC)

## Definition

A Network Interface Card (NIC) is a hardware component that allows a computer to connect to a wired or wireless network.

## Features

- Operates at Layer 1 (Physical) and Layer 2 (Data Link).

- Has a unique 48-bit MAC Address assigned by the manufacturer.

- Available in Ethernet (wired) and Wi-Fi (wireless) versions.

- Converts digital data into electrical, optical, or wireless signals.

## Advantages

- Enables communication between computers.

- Provides high-speed network connectivity.

- Supports both wired and wireless networking.

- Does not reduce network congestion.

# 2. Repeater

## Definition

A Repeater is a networking device that regenerates weak signals and retransmits them over longer distances.

## Features

- Operates at Layer 1 (Physical).

- Regenerates weak signals.

- Does not inspect data or addresses.

- Usually has two ports.

## Advantages

- Extends network coverage.

- Improves signal quality.

## Disadvantages

- Cannot filter traffic.

- Does not reduce network congestion

.

3. Hub

Definition

A Hub is a multi-port repeater that broadcasts incoming data to all connected devices.

Features

- Operates at OSI Layer 1 (Physical Layer)

- Broadcasts data to every connected device

- Single collision domain

- Single broadcast domain

- Supports half-duplex communication

Advantages

- Low cost

- Easy to install

- Simple to use

Disadvantages

- Causes frequent network collisions

- Low performance

- No security or traffic filtering

4. Bridge

Definition

A Bridge connects two LAN segments and forwards frames using MAC addresses.

Features

- Operates at OSI Layer 2 (Data Link Layer)

- Learns MAC addresses

- Filters unnecessary traffic

- Reduces collisions

- Connects two LAN segments

Advantages

- Improves network performance

- Reduces unnecessary traffic

- Decreases collisions

Disadvantages

- Supports only a limited number of LAN segments

- Slower than modern switches

- Becoming less common

5. Switch

Definition

A Switch is a multi-port bridge that forwards data only to the destination device using MAC addresses.

Features

- Operates at OSI Layer 2 (Data Link Layer)

- Uses a MAC Address (CAM) Table

- Supports full-duplex communication

- Each port has its own collision domain

- Single broadcast domain (unless VLANs are configured)

Types of Switches

- Unmanaged Switch

- Managed Switch

- PoE (Power over Ethernet) Switch

- Layer 3 Switch

Advantages

- High-speed communication

- Reduces collisions

- Efficient bandwidth utilization

- Improves network performance

Disadvantages

- More expensive than hubs

- Managed switches require configuration

- Broadcast traffic still exists without VLANs

6. Router

Definition

A Router connects different networks and forwards packets using IP addresses.

Features

- Operates at OSI Layer 3 (Network Layer)

- Uses IP addresses for routing

- Maintains routing tables

- Performs Network Address Translation (NAT)

- Connects LANs to WANs or the Internet

Advantages

- Connects multiple networks

- Separates broadcast domains

- Provides Internet connectivity

- Supports security features like ACLs and firewalls

Disadvantages

- Higher cost

- More complex configuration

- Slower than switches for local traffic

7. Gateway

Definition

A Gateway connects networks using different communication protocols and acts as a protocol translator.

Features

- Operates across multiple OSI layers (commonly Layer 7)

- Performs protocol conversion

- Connects different network architectures

- Often integrated with routers

Advantages

- Enables communication between different protocols

- Supports protocol translation

- Allows interoperability between dissimilar networks

Disadvantages

- Expensive

- Complex to configure and maintain

Comparison of Network Devices

| Device | OSI Layer | Forwarding Based On | Collision Domain | Broadcast Domain |
| --- | --- | --- | --- | --- |
| Hub | Layer 1 | Signal | One (Shared) | One |
| Bridge | Layer 2 | MAC Address | One per Segment | One |
| Switch | Layer 2 | MAC Address | One per Port | One |
| Router | Layer 3 | IP Address | One per Port | One per Interface |

Local Area Network (LAN)

Definition

A Local Area Network (LAN) connects computers and devices within a limited geographical area such as a home, office, school, or campus.

Characteristics

- High-speed communication

- Low latency

- Privately owned

- Reliable

- Secure

- Easy to maintain

LAN Topologies

1. Star Topology

All devices are connected to a central switch or hub.

Advantages

- Easy to install and manage

- Failure of one device does not affect others

- Easy troubleshooting

Disadvantages

- Failure of the central switch stops the network

- Requires more cable

2. Bus Topology

All devices share a single backbone cable.

Advantages

- Low installation cost

- Simple design

Disadvantages

- Backbone failure affects the entire network

- Difficult to troubleshoot

- Performance decreases with more devices

3. Ring Topology

Devices are connected in a circular loop.

Advantages

- No data collisions (using token passing)

- Equal access for all devices

Disadvantages

- Failure of one device or cable can break the network

- Difficult to expand

4. Mesh Topology

Every device is connected to every other device.

Advantages

- Highly reliable

- Multiple communication paths

- Excellent fault tolerance

Disadvantages

- Expensive

- Complex installation

- Requires many cables

LAN Components

| Component | Purpose |
| --- | --- |
| Switch | Connects computers in the LAN |
| Ethernet Cable | Transfers data |
| RJ45 Connector | Connects Ethernet cables |
| NIC (Network Interface Card) | Connects a computer to the network |
| IP Address | Identifies each device |
| Subnet Mask | Identifies the network and host portions |

Private IP Address Ranges

| Class | Range | Default Subnet Mask |
| --- | --- | --- |
| A | 10.0.0.0 – 10.255.255.255 | 255.0.0.0 |
| B | 172.16.0.0 – 172.31.255.255 | 255.255.0.0 |
| C | 192.168.0.0 – 192.168.255.255 | 255.255.255.0 |

Example LAN Configuration

| Computer | IP Address | Subnet Mask |
| --- | --- | --- |
| Computer 1 | 192.168.1.10 | 255.255.255.0 |
| Computer 2 | 192.168.1.11 | 255.255.255.0 |
| Computer 3 | 192.168.1.12 | 255.255.255.0 |

Testing Connectivity – Ping Command

Definition

The ping command tests connectivity between two devices on a network.

Syntax

ping <IP_address>

Example

ping 192.168.1.11

If the destination device is reachable, the command returns Reply messages along with the Round-Trip Time (RTT).

Advantages of LAN

- High-speed data transfer

- Easy resource sharing

- Cost-effective

- Centralized management

- Improved communication

- Easy maintenance

Disadvantages of LAN

- Limited geographical coverage

- Initial installation cost

- Requires maintenance

- Security risks if not properly configured

Conclusion

Network devices such as NIC, Repeater, Hub, Bridge, Switch, Router, and Gateway play a vital role in computer networking. Among these, the Switch is the most widely used device in modern LANs because it forwards data efficiently using MAC addresses, reducing collisions and improving performance. A LAN provides fast, reliable, and secure communication within a limited area, making it ideal for homes, schools, offices, and organizations.

Corrections made:

- Corrected "Router: Broadcast Domain = Per Port" to "Per Interface" (more accurate terminology).

- Added a note that a switch has a single broadcast domain unless VLANs are configured.

- Improved grammar, consistency, and exam-friendly formatting.

- Added a few important points (security, bandwidth utilization, fault tolerance, troubleshooting) for more complete notes.